COMPUTER NETWORKS — CCP PROJECT

WEEK 4 STATUS REPORT

Problem 1: Multi-Site Enterprise Network

| Report Date | June 9, 2026 (Monday) |
| --- | --- |
| Reporting Period | Week 4 — Final Testing & Documentation |
| Submitted By | Zohaib Ahmad (PM & Network Architect) |
| GitHub Repository | https://github.com/ZohaibAhmad260/CCP-CNS2026-Problem1 |

1. Executive Summary

Week 4 marks the completion of all core technical deliverables for the CCP-CNS2026 Problem 1 project. The team successfully conducted comprehensive network connectivity and security testing across all three VLANs (Sales VLAN 10, Engineering VLAN 20, Admin VLAN 30). All 12 test cases passed, confirming that inter-VLAN routing, ACL-based security policies, DHCP assignment, and DoS mitigation are functioning correctly. Final GNS3 project files, running configurations, and all documentation have been prepared for GitHub submission.

2. Team Contributions — Week 4

| Team Member | Role | Week 4 Contribution |
| --- | --- | --- |
| Zohaib Ahmad | PM & Network Architect | Executed all ping tests, verified ACL behavior, managed GNS3 project save, coordinated deliverables |
| M. Efan Amir | Security Engineer | Verified ACL hit counts (BLOCK_DOS_ATTACKER: 45 hits), confirmed security policies active |
| Muhammad Abdullah | Automation Lead & QA | Compiled test matrix, validated pass/fail results, QA sign-off |
| Tayyab Hassan | Documentation Lead | Authored Week 4 status report, updated GitHub README, organized all deliverable files |

3. Week 4 Completed Tasks

| Status | Task | Details |
| --- | --- | --- |
| ✅ | Ping Performance Tests | Ran 6 inter-VLAN ping tests + gateway test across PC1, PC2, PC3. All results recorded with latency values. |
| ✅ | ACL Verification | Confirmed BLOCK_SALES_TO_ADMIN and BLOCK_DOS_ATTACKER ACLs active and enforcing correctly. ICMP type:3, code:13 responses verified. |
| ✅ | Final Test Matrix | Completed 12-test-case matrix with pass/fail results, latency data, and ACL behavior documentation. |
| ✅ | Running Config Saved | R1 running configuration exported as R1_running_config.txt for all VLANs, DHCP pools, and ACLs. |
| ✅ | GNS3 Project v3 Saved | Final topology saved as CCP_Problem1_Week4.gns3 on E: drive. |
| ✅ | Week 4 Status Report | This document — covering all completed and pending deliverables. |
| ✅ | GitHub Push | All Week 4 files pushed to repository with commit message "Week4: Final testing complete, all 12 TCs passed". |

4. Test Results Summary

Total Test Cases: 12     Passed: 12     Failed: 0     Pass Rate: 100%

Key Findings:

Sales VLAN (PC1) is correctly isolated from Engineering and Admin VLANs by BLOCK_SALES_TO_ADMIN ACL.

DoS mitigation ACL (BLOCK_DOS_ATTACKER) successfully blocks all ICMP traffic from attacker IP 192.168.10.11.

Engineering (PC2) and Admin (PC3) VLANs can communicate bidirectionally with average latency under 70ms.

All three DHCP pools are functioning correctly — PCs retain correct IP assignments.

Gateway reachability confirmed for VLAN 20 (192.168.20.1, avg 28.3ms).

SSH access to R1 remains operational (verified in Week 2, not changed in Week 4).

5. Known Limitations & Mitigations

| Limitation | Reason | Mitigation / Documentation |
| --- | --- | --- |
| DHCP Snooping not configured | GNS3 built-in Ethernet Switch does not support the command | Documented in report. ACLs provide equivalent protection. |
| Kali/Ubuntu VMs not used | C: drive storage constraints on host machine | VPCS used for DoS simulation. Attack and mitigation successfully demonstrated. |
| First ping packet timeout | ARP resolution delay in GNS3 environment | Expected behavior. Subsequent packets succeed. Noted in test matrix. |

6. Week 5 Deliverables Plan (Tuesday)

| Deliverable | Assigned To | Target |
| --- | --- | --- |
| Final Report (15-20 pages) | Tayyab Hassan + Zohaib Ahmad | Tuesday AM |
| Video Demonstration (8-10 min, YouTube unlisted) | Zohaib Ahmad | Tuesday PM |
| Presentation Slides (12 min) | All team members | Tuesday PM |
| Incident Response Report (3-5 pages) | M. Efan Amir | Tuesday AM |
| Peer Evaluation Forms | All team members individually | Tuesday PM |

Week 4 Status: COMPLETE ✓ | All technical deliverables done | Ready for Week 5 final submission
CCP-CNS2026 Problem 1 | Deadline: Wednesday 8:00 AM